Multi Layered Air Traffic Control System using Poisson Processes

K. Nishitha (BL.EN.U4CSE23020)#1 , N. S. Kavya (BL.EN.U4CSE23032)#2, , V. Smrithika (BL.EN.U4CSE23059)#3   , Dr B Venkatesh#4

#Department of Computer Science of Engineering, Amrita School of Engineering, Bangalore, India-560035

1bl.en.u4cse23020@bl.students.amrita.edu

2bl.en.u4cse23032@bl.students.amrita.edu

3bl.en.u4cse23059@bl.students.amrita.edu

4b_venkatesh@blr.amrita.edu

Abstract— The paper introduces a novel approach to enhancing flight safety based on Multi-Poisson Layer Modelling (MPLM) which combines the monitoring of the air traffic and weather for the prediction and prevention of mid-air collisions. At each altitude level, aircraft is a Poisson process, and aircraft may move up and down accordingly with a compound Poisson process. The system monitors weather and risk collision, notifying both pilots and Air Traffic Control (ATC) if any dangerous conditions or hazards are detected, the system produces safe detours in order to prevent accidents and bad weather. This technique improves safety by providing real-time dynamic rerouting using air traffic as well as environmental conditions. It offers the possibility of a scalable solution for the future urban air traffic as well as for current air traffic.

Keywords— multi-poisson layer modelling, air traffic monitoring, weather integration, mid-air prevention ,poisson process, compound poisson process, real time rerouting , flight safety, air traffic control.

I  INTRODUCTION

The referred paper [1] authored by John Hannah, Robert F. Mills, Richard A. Dill, Douglas D. Hodson, discusses the feasibility of fake injection attacks on Traffic Collision Avoidance Systems (TCAS), determining their possible impact and feasibility for aviation safety. TCAS is an essential system to avoid mid-air collisions, but its vulnerability to cyber-attacks creates serious security issues. The research illustrates that specially designed fake injection attacks can be used to manipulate TCAS behaviour, causing dangerous aircraft movement. Experimental findings emphasize potential threats from these attacks and require enhanced security systems. To be secure, TCAS needs vigorous countermeasures, such as stronger authentication to avoid unauthorized handling of data. These defences should be fortified for the reliability and safety of air traffic management to be ensured

The referred paper [2] authored by Kaya Kuru focuses on developing a mid-air collision-free trajectory for

autonomous aircraft, ensuring safe navigation in congested airspace. Autonomous trajectory planning is essential for preventing collisions, especially with the increasing adoption of unmanned aerial vehicles. The study presents a novel approach to optimizing flight paths, minimizing conflicts while maintaining operational efficiency. Experimental results demonstrate the effectiveness of the proposed method in avoiding collisions under various flight conditions. The findings highlight the importance of real-time adaptive strategies in autonomous air traffic management. Future work aims to enhance system robustness by integrating AI-driven decision-making and dynamic environmental adjustments.

The referred paper [3] authored by Canqiang Wenga, Can Chena, describes a real-time traffic simulation and management system suitable for large scale urban networks that utilizes algorithms and data-driven approaches to optimize traffic and enhance transportation efficiency while reducing congestion. As urbanization continues to grow, traffic congestion and mobility challenges have emerged as a major nuisance for people worldwide, affecting their daily lives as well as being an important player in the economic productivity of a city. The purpose of this research is to evaluate and deploy innovative data systems for providing better approaches for urban mobility utilizing simulation models, real-time data, and decisions supported by artificial intelligence. The authors deployed the proposed system in a live large-scale urban setting and found improvements in traffic efficiency, reductions in congestion, and improved engagement with real-world infrastructure conditions. When comparative analysis was done with existing systems it showed substantial performance gains without sacrifice. This type of experience is an important component for demonstrating the advantage of real-time data driven methodologies. In conclusion, the authors believe that real-time traffic management will become the norm for facing urban traffic challenges through more scalable and more efficient computational representations and models.

The referred paper [4] authored by Jan-P.Calliess, Michael Osbrone Stephen J. Roberts presents a conservative collision prediction and avoidance system designed to improve safety in dynamic environments. Using predictive modelling and real-time decision-making, the system diminishes collision risks while providing efficient operational performance. The research circumvents the constraints of existing methods by using robust algorithms that determine potential collision threats, influence trajectories, and avoid collision. Experimental results confirm the efficiency of this approach in limiting collision threats, improving navigational precision, and creating smoother transitions in more complex environments. A comparison indicates enhanced performance in these areas versus traditional methods. Overall, this research highlights the importance of predictive collision avoidance methods to improve autonomous navigation and transportation safety.

The referred paper [5] authored by Luis E. Alvarez ,Michael owen, Samuel Wu , Marc Brittain, Anushuman Das compares the collision risk and operational consequence of speed modifications in dynamic situations. It evaluates how speed fluctuations influence safety, efficiency, and system performance in general. Applying predictive modelling and simulation methods, the research analyses various scenarios for speed adjustments in order to avoid risks and achieve optimal operations. The findings confirm that managed speed modifications can remarkably lower collision risks without compromising traffic or operational efficiency. Comparative analyses point out the advantages of proactive speed management to enhance safety and performance. Generally, the study underscores the need to balance speed adjustments to promote both safety and operational efficiency.

The referred paper [6] authored by Sreebha Bhaskaran introduces a new algorithmic model of drone delivery management optimization that combines collision detection, avoidance, and resource allocation. The introduced Collision Detection and Avoidance (CDA) algorithm combines path planning and altitude reservation to avoid collisions in the air, increasing safety and efficiency in operation. Dijkstra, A\, and Bellman-Ford algorithms are used in the system for assessing transport safety, productivity, and the use of resources. Performance results illustrate Dijkstra's performance in low-order cases, whereas A\ is optimal for heuristic-based routing. Bellman-Ford, despite its utility in negative weights, reflects poorer performance. An average efficiency, reflected by a hybrid Ant Colony + A\* approach, is observed. The system effectively solves drone navigation in dynamic environments with emphasis on collision avoidance and energy-efficient routing. Future research will focus on proactive collision avoidance and application on large-scale simulations.

The referred paper [7] authored by Mihaela Mitici and Henk A. P. Blom aim to 1) present the mathematical collision probability estimation approaches for air traffic conflict and definitions.2) to outline the advances in estimating the probability of air traffic conflict and collision using a unified mathematical framework. 3) To various air traffic applications and their use of directed mathematical models for air traffic conflict and collision probability estimation; and 4) to provide insight into the capabilities and restrictions of the mathematical models in the evaluation of future ATM designs. It provides a unified mathematical framework for a better understanding of the capabilities and limitations of existing models. This study gives an overview of ATM as CPSS and the design challenges involved including continuous-time stochastic hybrid system modelling and interactions complexities. Models originally focused on current air traffic operations, but increasingly turned towards designs for ATM of the future. While there have been great strides in this area, a number of models are still having difficulty handling extremely rare events and multi-aircraft conflicts. The Reich model and others like it are becoming antiquated, whereas Monte Carlo(MC) . A special focus is placed on agent-based modelling (ABM) and hybrid Petri nets to tackle the complexities of socio-technical systems. Finally, we argue that early-stage safety analysis is essential and that the development of MC simulations, sensitivity analysis, and ABM are encouraged for safety analysis of advanced ATM designs.

This referred paper [8] authored by G.J. Bakker & Henk A. P. Blom addresses the validation of future Air Traffic Management (ATM) concepts, in particular, assessing collision risk in complex air traffic systems using mathematical models. Specifically, the intermediate goal is to build a model for collision risks between controlled aircraft in general route networks, and produce numerical results which are comparable with the classical Reich model of uncontrolled aircraft. The methodology starts with the first hitting of a Markov process to an absorbing boundary and then generalizes the classical boundary-crossing theory to include transient boundaries. Such an extension is motivated by the difficulty of solving boundary-valued partial differential equations (PDEs) for multiple aircraft in a network. The study also finds collision probabilities and rates based on general assumptions (including the Poisson distribution of aircraft in each sector) and compares the outcomes of this study with those of the Reich model. As a standard in the ancillary cost function, the paper explains its benefits in the context of getting intractable boundary-valued PDEs for large-scale air traffic networks, with multiple aircraft. This extended model offers a more general framework than the existing models for assessment of conflict risk and is directly applicable to 3D route networks, which has relevance for the validation and design of future ATM systems.

The referred paper [9] authored by Kyle A. Smith introduces a real-time air collision avoidance algorithm for multiple UAVs, operating in aerial space. The algorithm relies on the CPA geometry and is a reactive navigation-based collision avoidance algorithm. It allows to reduce its computational load by projecting the safety boundary concept on a finite path angle domain making it applicable for real-time applications. The method takes care of multiple intruders by sampling the safety boundaries, leading to an efficient collision detection and resolution. Also, the algorithm tackles the traditional geometric approaches well encountering with the singularity problems that might lead to out-of-control movements. The system is decentralized and allows non-cooperative UAVs to prevent collisions effectively. Numerical simulations prove the efficiency of proposed method within complex scenarios including cooperative and non-cooperative intruders. The method may as well be integrated to path planning algorithms on a global level to avoid collision while on midair.

The referred paper [10] authored by Dongbin LI and Xiaohao XU introduced a system collision risk model for air traffic control (ATC) that incorporates defensive barriers, human factors, and the impact of newer technologies such as data link communication, satellite based navigation and radar surveillance. This model is different from previous ones in that it is based on a systemic approach (using a collision event tree to analyses collision risk) and takes into account all major factors that influence risk, including communication, navigation and surveillance (CNS) systems as well as collision avoidance systems such as STCA and TCAS. The model aims to tackles some of the issues that the growing traffic flow and collision risk in China’s airspace. The new model not only builds on the outdated Reich Model, but also introduces modern conflict management measures, like strategic conflict management and separation provision, to mitigate the risk of collision. A systemic method is key to evaluating aircraft separation standards, safety, and the effectiveness of collision avoidance systems, according to the paper. The introduced model itself serves as a flexible base for advances in ATC systems in terms of enhanced automation and safety, emphasizing the need for empirical data and scientific refinement.

This referred paper [11] authored by James P. Chryssanthacopoulos and Mykel J. Kochenderfer  explores improving airborne collision avoidance systems by incorporating probabilistic pilot response models. Traditional TCAS relies on deterministic models that do not account for variability in pilot behavior, potentially degrading safety. The study formulates the problem as a Markov Decision Process (MDP) and optimizes the logic using dynamic programming (DP) with different pilot response models—linear, quadratic, and deterministic. Simulation results from half a million encounters show that DP-based logic outperforms TCAS by resolving more conflicts with fewer alerts while being more robust to unmodeled pilot behavior. Probabilistic models demonstrate lower Near Mid-Air Collision (NMAC) rates and reduced sensitivity to deviations from expected pilot responses. The findings suggest that incorporating probabilistic behavior improves safety and efficiency across different operating environments. Future work includes integrating sensor noise, transforming the problem into a Partially Observable MDP (POMDP), which requires advanced approximation techniques for effective decision-making.

This referred paper [12] authored by Hyunjin CHOI and Youdan KIM addresses the midair collision (MAC) problem for multiple unmanned aerial vehicles (UAVs) in dynamic airspace. While existing collision avoidance systems, such as TCAS, have reduced MAC risks for civil aircraft, UAVs require new approaches due to their increasing use. Various collision avoidance methods from robotics, including path planning and reactive collision avoidance, have been explored. However, many prior methods focus on single aircraft or cooperative scenarios. The study proposes a multiple UAV collision avoidance algorithm based on sensor-based detection using the closest-point-of-approach (CPA) geometry. By estimating intruder positions and velocities, the algorithm determines potential collisions and reacts accordingly. A finite path angle domain and safety boundary sampling are introduced to reduce computational load and allow real-time execution. The method also addresses singularity issues in geometric approaches, preventing erratic maneuvers. The decentralized nature of the algorithm makes it suitable for non-cooperative "Sense-and-Avoid" scenarios. Simulations demonstrate its effectiveness in avoiding collisions for both cooperative and non-cooperative intruders, making it adaptable to various MAC situations and combinable with global path planning strategies.

The paper [13] authored by V Sowbaranic Raj, Jalakam Venu Madhava Sai, N A Lakkshmi Yogesh  an intelligent traffic control system intended to identify and give priority to Emergency Vehicles (EmVs) at jammed intersections based on video and audio processing. YOLO V5 is utilized to detect EmVs in real-time video through CCTV footage, whereas a VGGNet-based Convolutional Neural Network with a linear classifier is applied to identify siren sounds. The innovation of the paper is in the prioritization of EmVs on the basis of their speed, so that the quickest vehicle gets signal clearance prior to others. For training the video model, a dataset of 500 images was utilized with a mean Average Precision (mAP) of 0.824 and accuracy of 71%. The audio model, trained on more than 8500 siren audios, had a validation accuracy of 85%. A distinct class for non-EmVs minimized false positives due to visually similar vehicles. The combination of video and audio modules pushed the overall detection rate up to 88%. The dual-modality strategy worked particularly well under noisy and low light conditions. The proposed model outperforms the current approaches and provides a trustworthy means of real-time EmV prioritization at traffic lights.

This paper [14] authored by Vedansh Bharadwaj ,Yaswanth Rasamsetti,Vipina Valsan introduces a real-time traffic control system based on image processing to minimize congestion in smart cities. The system utilizes Canny Edge Detection in MATLAB to identify vehicle density in individual lanes. Depending on traffic density, green signal time is dynamically changed to prevent delay. It provides priority to emergency vehicles such as ambulances by immediately changing their lane signal to green. The model is simulated and validated with the help of Proteus software. It largely enhances signal response time and lessens idle green time during empty roads. Yet, the system falters in foggy weather as a result of low image clarity. At present, it can only detect ambulances, indicating potential for future enhancement.

Although the majority of systems are sophisticated, The MPLM is the approach used in this system to coordinate safety of flights by considering air traffic and weather information. The movement of aircraft is modelled by an independent Poisson processes for each altitude level, while vertical changes and potential collisions between levels are represented by a compound Poisson process. Flight information evaluate the risk of collision, and weather information (rain, thunderstorm, etc.) is integrated to evaluate the environmental safety of aerial connections in the given situation. After all, the system only allows a flight to go forward after clearing two key hurdles, collision risk and weather severity. If the above two checks are successful, the flight is authorized with a message labelled "safe to proceed". But if danger is imminent  if there’s a good chance of a midair collision, or of even riskier weather to come  the system and moves the airplane to a safer course. During the rerouting operation, the alternative route is planned with consideration of traffic level, weather risk and fuel economy, and the newly planned route is sent to the pilot and ATC simultaneously. The flight plan is constantly updated and modified if needed to ensure a safe and optimal flight through challenging airspace.

II  METHODOLOGY

- State space for each altitude layer:

For each layer i  at the time t , the state of the system is defined by the set of aircraft positions within that layer:

Si(t)={  xi,1t,yi,1t,zi, xi,2t,yi,2t,zi, .. , xi,Ni(t)t,yi,Ni(t)t,zi}

Ni(t) : number of aircraft present in layer  i  at time t.

(xi,j(t),yi,j(t),zi) : 3D coordinates of the  jth -th aircraft in layer i at time  t .

zi: it is the fixed for each layer, representing the constant altitude specific to that layer.

1.Poisson processes for each layer:

The number of aircraft arriving in altitude layer i during a time interval  ∆t follows a poisons distribution with parameter λi*∆t :

P(Ni(∆t=k))= (⋋i∆t)k  * e-(⋋i∆t) / k ! ,

k=0,1,2,3,...

Ni(∆t) :  number of aircraft in layer i within time interval  ∆t

λi  :  arrival rate for layer i (average number of aircraft entering per unit time).

2.Spatially varying arrival rate:

If the arrival rate varies spatially (to model traffic density differences), we denote the rate as  λi(x,y):

λi(x,y)  is the arrival rate at position (x,y) in layer i.

The expected number of arrivals in a region A within layer i over time ∆t is:

E[ Ni(∆t,A)]=A[⋋ix,y dxdy]*∆t

For our initial simulation, we assume a constant arrival rate  λi across the entire layer, simplifying the model as:

E [Ni (∆t)]=λi* ∆t*|A|

Where |A| is the area of the region within the layer.

- Compound poissons process for vertical transition

1.Transition rate definition:

γi⟶j: rate of transition  from layer i to layer j ( i ≠j) .

Γ : transition rate matrix , where :

Γij =  γi→j   for i ≠j .

Γii = - j≠iγi→j  (total rate of aircraft leaving layer I ).

Γ.    =       -j≠1γ1→j γ1⟶2γ1⟶3γ2⟶1 j≠2γ2→j γ2⟶3γ3⟶1γ3⟶2 j≠3γ3→j      ⋯⋯⋯

⋮.                        ⋮.                     ⋮.     ⋱

2.Modelling the vertical transition process:

Number of transitions from layer i to layer  j :

The number of transitions from layer i to layer j  during ∆t , given Ni(∆t) aircraft in layer i , follows a poisons distribution with rate:

P(Transitions i ⟶j)=([ Ni(∆t)γi→j ∆t] k  *  e( Ni(∆t)γi→j ∆t))/ k!

For small ∆t , the probability of individual aircraft transitioning from layer  i to layer  j is approximately:

P(Aircraft from i⟶j)=  γi→j (∆t)

3.Total transition rate from a layer:

If a total departure rate from layer i is given by :

γi=j≠iγi→j

If a transition from layer i occurs , the probability of moving to layer j is:

pi→j=γi→j γi

γi→j= γi*pi→j

- Probability of Collision:

1.Collision within the same layer:

For two aircraft j and k within the same altitude layer i at the time t, a collision is defined as:

(xi,jt-xi,kt)^2+(yi,jt-yi,kt)^2   ≤dsafe

Define a binary decision variable   Ci,j,kt :

Ci,j,kt 1     if    (xi,jt-xi,kt)^2+(yi,jt-yi,kt)^2 ≤ dsafe0    otherwise

Collision probability (same layer): The collision probability within layer i at time t is given as the expected number of collisions pairs.

pcol(i)=E[  j=1Ni(t).k=j+1Ni(t)Ci,j,kt  ]

Variables for Collision within the Same Layer:

i: Index of altitude layer, ranging from i ϵ {1,2,..,i}.

j,k: Indices of aircraft in layer i, such that j,k ϵ {1,2,..,Ni(t)} and j≠k.

Nit: Number of aircrafts present in layer i at time t.

(xi,jt,yi,j(t)): Horizontal coordinates of aircraft j in layer i at time t.

dsafe: Minimum safe separation distance required to avoid collision.

Ci,j,kt: Binary collision indicator variable between aircraft j and k in layer i at time t.

Ci,j,kt:  1  if distance  ≤ dsafe                  0   otherwise

Pcol(i): Expected number or probability of collision events within layer i at time t.

2.Collision between different layers:

Collision occurs when an aircraft transitioning from layer i to layer j is within dsafe of another aircraft in layer  j at the transition time:

(xi,mt-xj,n(t))2+(yi,mt-yj,n(t))2≤dsafe

Binary decision variable Ci→j,m,nt:

Ci→j,m,nt=1   if   (xi,mt-xj,n(t))2+(yi,mt-yj,n(t))2≤dsafe                          0        otherwise

Collision probability:

Between the layers layer i and j at time t considering the transition rate γi→j.

Pcol(i→j):m=1Ni(t)n=1Nj(t)Ci→j,m,nt.γi→j

i: Index of altitude layers involved in transitions, i≠j.

m: Index of aircraft in the originating layer i,m ϵ 1,2,..,Nit.

n: Index of aircraft in the originating layer i,n ϵ 1,2,..,Njt.

Nit,Njt: Numbers of aircraft in layers layer i to layer j at time t.

(xi,mt,yj,m(t): Historical coordinates of aircraft m in layer i at time t.

(xi,nt,yj,n(t): Historical coordinates of aircraft n in layer i at time t.

dsafe:  Minimum safe separation distance.

Ci→j,m,nt: Binary collision indicator for collision during vertical transition.

Ci→j,m,nt=1   if distance ≤dsafe                                            0   otherwise

γi→j: Transition rate from layer  i to layer j.

Pcol(i→j): Expected number of collision events during vertical transitions.

- Defining a high-risk scenario:

high_risk ϵ {0,1}

high-risk1  if(mindistance< dthreshold) or (weatherhazard> wthreshold0                    otherwise

min_distance: minimum distance from the given aircraft to any other nearby aircraft.

dthreshold: Distance threshold to define risk.

weatherhazard: A continuous score in [0,1] representing the severity.

wthreshold: weather hazard threshold.

- Formulation for Rerouting criteria:

Rtϵ{0,1}

Rt=1   if high_riskt=1 0    otherwise

Rt:rerouting decision where 1 indicates rerouting is triggered

- Evaluating Alternative Routes:

R={R1,R2,..,RM}

CsafetyRm: predicted collision risk score along route Rm.

CweatherRm: Aggregate weather hazard score along route Rm.

CfuelRm: fuel cost.

CtrafficRm: Traffic density along Rm.

CdeviationRm: Measure of deviation from the original path

xm:1      ifRm is selected        For m =1,....,M      0     otherwise

1.Weighted Cost:

We aim to select the route with minimum total cost:

minm=1Mxm .  [ w1Csafety Rm+w2Cweather Rm+w3CfuelRm+w4Ctraffic Rm+w5Cdeviation Rm]

m=1Mxm=1 ,  xm∈{0,1}

xm: Binary variable: 1 if route Rm selected, else 0

Csafety Rm: Predicted collision risk along route Rm

Cweather Rm: Aggregate weather hazard score along Rm

CfuelRm: Fuel cost proxy for Rm

CtrafficRm: Traffic density or congestion along Rm

Cdeviation Rm: Deviation measure from original flight plan

w1,w2,w3,w4,w5: Weight coefficients representing the importance of each factor

- MPLM Integration as Features

1.MPLP Collision Risk Score

At time ti, foe aircraft j in layer I, define the MPLM-derived collision risk as:

Ri,jt=Pcollision involving aircraft j in layer i at time t

This probability is derived from the Multi-Poisson Layer Model using aircraft density, arrival rates λi, transition rates γi→k and traffic movement patterns.

2.Feature Vector Augmentation:

Given the original feature vector for aircraft j at time t:

Xi,jt=[x1,x2,. . . ,xm]

Augment it by adding the MPLM risk score:

Xi,j't=[x1,x2,. . . ,xm,Ri,jt]

3. Machine Learning Model Input:

The ML model then uses the augmented feature vector Xi,j't to predict the target risk label yi,jt:

yi,j^t= fθ(Xi,j't)

where  fθ is the ML model parameterized by θ

Ri,jt: MPLM-derived collision risk for aircraft j in layer i at time t

Xi,jt: Original feature vector for aircraft j at time t

Xi,j't: Augmented feature vector including MPLM risk

yi,j^t: ML model prediction for aircraft j at time t

fθ: Machine learning model function with parameters θ

- Formulation for Risk-Informed Rerouting

1.Predict Collision Risk for each grid cell:

For each grid call c at a time t, define:

Xct = feature vector for cell c at time t

Which may include:

Average traffic density in cell c,

Weather hazard score in cell c,

Other environmental or operational features.

Use the trained ML model fθ to predict risk:

Rct= fθ(Rct)

Where Rct∈[0,1] represents the predicted collision risk for cell c.

2. Dynamic Cost Grid

Define the dynamic cost of each cell c at time t as:

Costct=α. BaseCostc+ β.Rc(t)

where:

BaseCostc is the original cost

Rc(t) is the predicted collision risk for cell c,

α, β are weighting factors balancing base cost and risk.

3. Risk-aware Pathfinding

For a path P = {c1,c2,. . . ,cn}, defines total path cost as:

Cost(P)=k=1nCostck(t)

The rerouting algorithm seeks to find:

P*=arg minP Cost(P)

Which minimizes both traditional costs and predicted collision risk.

Xct:Feature vector for grid cell c at time t.

Rct:Predicted collision risk for cell c at time t.

BaseCostc:Base cost of cell c.

α, β:Weights balancing base cost and predicted risk.

Costc(t):Dynamic cost of cell c at time t.

P:Candidate path as a sequence of grid cells.

P*:Optimal path minimizing cost.

- System Evaluation:

1. Collision Avoidance Rate (CAR):

CAR=Number of avoided collisionsNumber of petential collisions

Number of avoided collisions: cases where system successfully prevented a collision.

Number of potential collisions: cases identified as high-risk by the baseline.

2. Efficiency of rerouting (ER):

ER=Deviation cost of new routeDeviation cost of original route

Deviation cost: Increased distance, fuel consumption, and time delay.

3.System Acceptance (SA):

SA=Number of accepted rerouting decisionstotal rerouting decisions

Number of accepted rerouting decisions: Scenarios where the rerouting was deemed acceptable by an evaluation criterion.

Function:

Define all the overall performance metric as a weighted combination of key evaluation metrices :

Performance = w1 * CAR - w2 * FAR - w3 * ER + w4*SA

Where :

w1,w2,w3,w4  are the weights representing the importance of each metric .

CAR = collision avoidance Rate

ER  = efficiency of rerouting .

SA = System Acceptance

key parameters and thresholds

Collision risk threshold for alert/rerouting (τ):

τ = minimum predicted risk to trigger an alert or reroute

weight for MPLM risk in ML model (γ):

γ=weight of MPLM risk score in the augmented feature vector .

Cost function weights (α,β) :

Costc(t) = α * BaseCostc+ β * Rc(t)

Weight balancing the objective function (w1,w2,w3,w4):

performance = max(w1,w2,w3,w4) [ w1 * CAR - w2 * FAR - w3* ER + w4 * SA ]

III  ALGORITHM

Step 1: Start with binomial probability

PXn=k= nk pk1-pn-k

Where k=0,1,..,n.

Step 2: Model Arrival probability p and function n:

t = Total time interval.

Lambda = Arrival rate per unit.

P= p [arrival in a block]

≈λduration of the block

≈λtn

PXn=k=nkλtnk (1-λtn)n-k

Step 3: Take limit as

P[k arrivals] =limn→∞P[xn=k]

P[X(t)=k] = limn→∞   n!n-k!k!    (λt)knk       1-λtnn-k

P[X(t)=k] =limn→∞n.n-1…(n-k1)n.n….(n) (λt)k  k!(1-λtn)n(1-λtn)k

= 1.(λt)kk!.e-λt               t>0, k=0,1,2,3….

PXt=K=(λt)kk!.e-λt

Optimization approach:

Grid search: test combination of parameter values within predefined range.

Bayesian optimization: efficiency explore the parameter   space based on previous evaluations.

Genetic algorithms: use evolutionary strategies to optimize the performance metric.

IV  RESULTS

Fig 1

In Fig 1, The graph illustrates the number of aircrafts in each layer over time. Layer 1, Layer 2, Layer 3 maintain relatively constant numbers throughout the observed period. Layer 1 has approximately 65 aircrafts, Layer 2 has around 60, Layer 3 maintains around 90 aircrafts. These consistent values indicate stable traffic distribution among the layers, without significant fluctuations.

Fig 2

In Fig 2, The histogram shows the distribution of collision occurrences over time. Initially, the number of collisions is minimal, but it increases gradually and peaks at around the 200th time step, reaching approximately 1750 collisions. The curve exhibits a rising trend with a sharp increase after the midpoint of the observation period.

Fig 3

In Fig 3, A synthetic weather hazard map was generated to model spatial variability in atmospheric hazards across 100 x 100 grid. As illustrated in Fig .3, the hazard scores range from 0(no hazard) to 1 (extreme hazard). the highest concentrations of hazards are located centrally, around coordinates (50,50), forming a high-intensity weather cell. The surrounding areas display significantly lower scores, tapering off to near-zero hazards’ values at the edges of the grid.

Fig 4

In Fig 4,The estimated risk across the airspace was computed by integrated multiple hazards contributed such as weather severity. aircraft density, and rerouting complexity into a unified cost/risk Fig 4 shows the spatial distribution of these risk scores over 100 x 100 airspace grid. The central region approximately between coordinates (40,40) and (70,70), exhibits moderately elevated risk scores in the range of 0.3 to 0.5. This region aligns with previously identified high weather hazard zones and moderately dense traffic layers. The outermost regions exhibit relatively lower risk values, mostly below 0.1. indicating safer routing potential. A singular outer with a notably high-risk score is observed in the bottom-left corner, likely due to compounding factor such as isolated congestion or extreme weather outlier data.

Fig 5

In Fig 5, shows the output of the greedy rerouting algorithm applied cost / risk grid. The grid is colour coded, with yellow indicating low risk(cost) areas and purple representing high-risk zones. A green dot marks the starting location at approximately (20,20), while a blue 'X' indicates the goal approximately (80,80). The red lines trace the computed rerouted path based on greedy strategy. The algorithm begins from the start point and the progresses by selecting adjacent cells with the lowest immediate cost. The initial path demonstrates this behaviour, as it moves in a direction that avoids higher-cost zones. However, the path does not traverse through the central blue region near (50,50), which represents a broader area of low risk.

Fig 6

In Fig 6, The greedy rerouting algorithm employed a risk-based cost function to navigate from origin to destination from the beginning. As indicated in the figure, the algorithm took a detour from the high risk region in the middle by choosing the slightly modified route through lower-risk region. This illustrates that the algorithm can reduce exposure to risk yet still advance towards the objective effectively.

V  DISCUSSION

In Fig 1, the stable distribution of aircrafts across the layers suggests that the air traffic control system effectively maintains traffic balance. However, the total number of collisions recorded is notably high (10851), which could indicate potential issues related to collision avoidance logic or insufficient layer separation. Further investigation into inter-layer traffic interactions and flight path overlap is necessary to reduce collision rates.

In Fig 2, the increasing collision rate suggests that as time progresses, either traffic density or movement complexity rises, leading to more frequent conflicts. The peak near the 200th time step might indicate a saturation point where the airspace becomes overly congested or where the rerouting and conflict resolution mechanisms fail to adapt efficiently. Optimizing the conflict avoidance algorithm and ensuring adaptive traffic management could mitigate this issue.

In Fig 3, the spatial distribution of weather hazards shows a convective system with a core of severe activity, impacting rerouting and airspace management. aircraft near the core require real time avoidance, while those in outer areas follow standard protocols Integrating weather hazard data with aircraft density via multi-Poisson layer modelling is essential for layers risk assessment. High – density air traffic in hazardous zones raises the risk of mid-air conflicts or emergency rerouting. The map not only visualises weather risks but also supports dynamic algorithms and air traffic deconfliction in real – time systems.

In Fig 4, The risk map integrates key operational data for data-driven flight planning and airspace management. The central high- risk zone, where elevated weather hazards and flight density coincide, validates the risk models robustness. Low risk perimeter zones are suitable for rerouting, while isolated high-risk areas highlight the need for localized risk assessment. The map supports adaptive rerouting to avoid high risk sectors while maintaining efficiency and time constraints.

This approach using multi-Poisson layer modelling, enhances multi factor airspace safety and can integrate real time data in the future.

In Fig 5, Greedy algorithms make decisions based on local information, often leading to suboptimal paths by deviating from low-cost routes. While efficient , they struggle in complex environments where risk varies , as they focus on immediate gains rather than long-term optimization, unlike A* or Dijkstra’s algorithms.

In Fig 6, The greedy rerouting method successfully incorporates risk-based cost metrics into path planning to facilitate safer navigation through the avoidance of high-risk zones even at the expense of longer paths. Its dynamic response to the risk environment makes it particularly valuable for safety-critical applications such as UAVs and driverless cars. Enhancements in the future may involve real-time refreshing and comparison to more sophisticated planning schemes.

VI  CONCLUSION

The Multivariate Poisson Layer Modelling (MPLM) framework improves flight safety over the existing volumetric and layered modelling techniques learning from real-time air traffic data and weather events. Aircraft at each altitude level are simulated by considering individual Poisson processes, and their vertical motions are managed as a compound Poisson process. It's this nested nature of the modelling that allows the risk of mid-air collisions to be measured precisely. The system assesses both collision risk and weather hazard to approve or continue flight. Only flights that pass both checks are categorized as “safe to proceed.” Dynamic rerouting is started for safety if danger is detected. Alternative routes take fuel economy, weather hazards and air traffic into account pilots and ATC are informed of the latest routes on a regular basis. In complicated airspace, MPLM facilitates proactive, flexible flight planning.

VII  FUTURE SCOPE

Robustness and flexibility of the aircraft traffic system can be substantially improved by future additions. Combining deep learning models, like CNN’s may improve collision prediction by learning intricate patterns from historical data. Real time optimization can be significantly enhanced through adaptive rerouting using reinforcement learning algorithms such as DQN to adapt to changing conditions. If multi model data fusion is applied, it allows for global situational awareness by integrating ADS-B with radar and weather data. Augmented human-machine partnership realized by voice controlled diverting.

VIII  ACKNOWLEDGMENT

We are truly grateful to Dr. B. Venkatesh for his constant support, valuable suggestions, and thoughtful guidance, all of which have been essential to the success of this project. His wide range of experience and thoughtful suggestions have significantly raised the standard and effectiveness of our work. We would also want to sincerely thank the Department of Mathematics, whose resources and academic climate provided a strong foundation for this research. Their support and encouragement have been crucial in enabling us to achieve our goals in this endeavour.

IX  REFERENCES

[1]	J. Hannah, R. Mills, R. Dill, and D. Hodson, “Traffic collision avoidance system: false injection viability,” Journal of Supercomputing, vol. 77, no. 11, pp. 12666–12689, Nov. 2021, doi: 10.1007/s11227-021-03766-9.

[2]	K. Kuru et al., “Toward collision-free trajectory for autonomous and pilot-controlled unmanned aerial vehicles,” Sep. 2023, doi: 10.1109/ACCESS.2023.3314504.

[3]	C. Weng, C. Chen, J. Tan, T. Pan, and R. Zhong, “Real-time Traffic Simulation and Management for Large-scale Urban Air Mobility: Integrating Route Guidance and Collision Avoidance,” Dec. 2024, [Online]. Available: http://arxiv.org/abs/2412.01235

[4]	J.-P. Calliess, M. Osborne, and S. J. Roberts, “Conservative Collision Prediction and Avoidance for Stochastic Trajectories in Continuous Time and Space.” [Online]. Available: www.ifaamas.org

[5]	S. M. Katz et al., “Collision Risk and Operational Impact of Speed Change Advisories as Aircraft Collision Avoidance Maneuvers,” Apr. 2022, [Online]. Available: http://arxiv.org/abs/2204.14250

[6]	P. Sriharsha, Y. Gowtham, G. T. Chowdary, and S. Bhaskaran, “Optimizing Drone Delivery Management: An Algorithmic Framework for Collision Detection and Avoidance with Resource Management,” in 3rd International Conference on Integrated Circuits and Communication Systems, ICICACS 2025, Institute of Electrical and Electronics Engineers Inc., 2025. doi: 10.1109/ICICACS65178.2025.10968590.

[7]	M. Mitici and H. A. P. Blom, “Mathematical Models for Air Traffic Conflict and Collision Probability Estimation,” IEEE Transactions on Intelligent Transportation Systems, vol. 20, no. 3, pp. 1052–1068, Mar. 2019, doi: 10.1109/TITS.2018.2839344.

[8]	B. Bakker and H. A. P. Blom, “WP14-4:lO Air Traffic Collision Risk Modelling.”

[9]	K. A. Smith, “Collision Avoidance System Optimization for Closely Spaced Parallel Operations through Surrogate Modeling,” 2013.

[10]	D. Li, P. Candidate, X. Xu, and X. Li, “COLLISION RISK ANALYSIS IN AIR TRAFFIC CONTROL,” 2007.

[11]	J. P. Chryssanthacopoulos and M. J. Kochenderfer, Collision Avoidance System Optimization with Probabilistic Pilot Response Models.

[12]	H. Choi, Y. Kim, Y. L. 1þ, E. Tai, and K. 2þ, “A Reactive Collision Avoidance Algorithm for Multiple Midair Unmanned Aerial Vehicles.”

[13]	V. S. Raj, J. V. M. Sai, N. A. L. Yogesh, S. B. K. Preetha, and R. Lavanya, “Smart Traffic Control for Emergency Vehicles Prioritization using Video and Audio Processing,” in Proceedings - 2022 6th International Conference on Intelligent Computing and Control Systems, ICICCS 2022, Institute of Electrical and Electronics Engineers Inc., 2022, pp. 1588–1593. doi: 10.1109/ICICCS53718.2022.9788119.

[14]	V. Bhardwaj, Y. Rasamsetti, and V. Valsan, “Image Processing Based Smart Traffic Control System for Smart City,” in 2021 12th International Conference on Computing Communication and Networking Technologies, ICCCNT 2021, Institute of Electrical and Electronics Engineers Inc., 2021. doi: 10.1109/ICCCNT51525.2021.9579787.